# Open Ended Lab 02

# Semester Spring 2025

CEL-120 Digital Logic Design

BCE -2A

Areeba Shah                                                     02-132242-013

Supervisors:

Course Teacher: Engr. Muhammad Irfan Ul Haq Malik

Lab Instructor: Engr. Zohaib Rehman

Table of Contents

Objective	1

Statement	2

Components	3

Procedure	4

Methodology	5

Circuit Diagram	6

OBJECTIVE:

# Design a digital MOD-60 counter that increments its value on the negative edge of the clock signal, functioning as a seconds timer. Once it reaches its maximum count of 59, it resets to zero and increments a second MOD-60 counter, which functions as a minutes timer. This cascading process continues until both counters reach their maximum values. The current count of minutes and seconds should be displayed using four seven-segment displays.

# STATEMENT:

1. Construct a MOD-60 counter circuit using ICs such as the 74LS160 or 74LS190 by combining MOD-10 and MOD-6 counters to implement both seconds and minutes counting.

2. Interface four 7-segment displays with BCD to 7-segment decoder ICs (e.g., 7447) to display the output in MM:SS format.

3. Design the circuit to count sequentially from 00:00 up to 59:59, and automatically reset back to 00:00 upon reaching the maximum count.

4. Build and simulate the complete circuit in NI Multisim to verify correct operation and proper display of the timer.

# COMPONENTS:

- 74LS160 – 4-bit Synchronous Binary Counters (used in pairs):

  - One pair configured to count seconds (MOD 60 = MOD 10 × MOD 6).

  - Another pair configured to count minutes (MOD 60 = MOD 10 × MOD 6).

- 7447 or 74LS47 – BCD to 7-Segment Decoder Drivers:

  - These ICs convert the BCD outputs from the counters into signals suitable for driving 7-segment displays.

- Four 7-Segment Displays:

  - Two displays for seconds (units and tens).

  - Two displays for minutes (units and tens).

- Pulse Generator or Clock Oscillator:

  - Generates a clock signal (typically 1 Hz) to drive the seconds counter.

- AND Gates or Combinational Logic Network:

  - Used to detect when a counter reaches the value 59, triggering a reset of the current counter and an increment in the next counter stage.

# PROCEDURE:

1. Launch NI Multisim and start a new blank schematic project.

2. Insert two 74LS160 counters to represent the seconds portion—one for the units digit and one for the tens digit.

3. Add two additional 74LS160 counters to handle the minutes—again, one for units and one for tens.

4. Connect the clock source to the seconds unit counter and cascade the carry-out signals appropriately to the next stage counters.

5. Interface each counter output with a 7447 BCD-to-7-segment decoder.

6. Connect four 7-segment displays to the outputs of the decoders, representing MM\:SS format.

7. Implement logic using AND gates to detect when the counters reach 59:59, and apply a reset signal if required.

8. Ensure all components are properly powered by connecting VCC and GND.

9. Run a simulation to confirm that the timer counts correctly from 00:00 to 59:59 and resets as expected.

# METHODOLOGY:

The Methodology to design the MOD 60 timer, two cascaded MOD 60 counters are implemented using synchronous BCD counter ICs such as the 74LS160 or 74LS190, with one counter dedicated to seconds and the other to minutes. Each MOD 60 sequence is achieved by cascading a MOD 10 (units place) and a MOD 6 (tens place) counter. The circuit is clocked using a low-frequency oscillator or a simulated 1 Hz clock source to increment once per second.

The seconds counter begins counting from 00 to 59, and upon reaching 59, it generates a carry-out pulse which is used to increment the minutes counter. Similarly, the minutes counter follows the same MOD 60 logic and resets after reaching 59. This cascading structure ensures that the timer accurately displays the range from 00:00 to 59:59. Both counter outputs are interfaced with BCD to 7-segment decoders (74LS47 or 7447), which then drive four 7-segment displays (two for seconds and two for minutes).

The complete design circuit is simulated on NI Multisim, where the functional behavior of each component is verified, including reset conditions, clock synchronization, and carry propagation. This simulation allows for real-time observation of the timer progression, ensuring proper logic implementation and visual confirmation of the “59:59” display

CIRCUIT DIAGRAM: